VHDL

C.1 LFSR VHDL code

C.2 MUX VHDL code

C.3 Clock_Driver VHDL code

C.4 Abs VHDL code

C.5 RD_EN VHDL code

C.6 BFSK_demodulator VHDL code

C.7 Test Bench VHDL code

C.1 LFSR VHDL code

---------------------------------------------------------------------------------------------------------------------

-- Company:  ATU

-- Engineer:  Ahmed M. Alfadhel

-- Create Date: 04/22/2019 03:39:00 PM

-- Module Name: LFSR3 - Behavioral

-- Revision 0.01 - File Created

-- Additional Comments: This LFSR works to generate 7 different states by a freq. of 500 HZ

---------------------------------------------------------------------------------------------------------------------

-- Library's

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.numeric_std.all;

use IEEE.math_real.all;

entity LFSR3 is

Port (

i_clk           : in  std_logic;

o_lsfr          : out std_logic_vector (2 downto 0)

);

end LFSR3;

architecture Behavioral of LFSR3 is

signal r_lfsr	: std_logic_vector(2 downto 0) := "100";

constant maxcount  : integer := 10156;

begin

o_lsfr <= r_lfsr;

LFSR_proc: process(i_clk)

variable i_en    : std_logic := '0’;

variable counter2 :unsigned(14 downto 0) := to_unsigned(0, 15);

begin

if(rising_edge(i_clk)) then

if(i_en = '1')  then

r_lfsr(2) <= r_lfsr(0) xor r_lfsr(1);

r_lfsr(1) <= r_lfsr(2);

r_lfsr(0) <= r_lfsr(1);

counter2 := (others => '0');

i_en := '0';

elsif(counter2 /=(maxcount - 1)) then

counter2 := counter2 +1;

else

i_en := '1';

end if;

end if;

end process LFSR_proc;

end Behavioral;

C.2 MUX VHDL code

---------------------------------------------------------------------------------------------------------------------------

-- Company: ATU

-- Engineer: Ahmed M. Alfadhel

-- Create Date: 04/22/2019 11:14:09 AM

-- Module Name: mux2 - Behavioral

-- Revision 0.01 - File Created

-- Additional Comments: Multiplexer Digital circuit

---------------------------------------------------------------------------------------------------------------------------

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity mux is

Port (

data_in : in std_logic_vector(2 downto 0);

gateway_in0 : IN STD_LOGIC_VECTOR(7 DOWNTO 0);

gateway_in1 : IN STD_LOGIC_VECTOR(7 DOWNTO 0);

gateway_in2 : IN STD_LOGIC_VECTOR(7 DOWNTO 0);

gateway_in3 : IN STD_LOGIC_VECTOR(7 DOWNTO 0);

gateway_in4 : IN STD_LOGIC_VECTOR(7 DOWNTO 0);

gateway_in5 : IN STD_LOGIC_VECTOR(7 DOWNTO 0);

gateway_in6 : IN STD_LOGIC_VECTOR(7 DOWNTO 0);

gateway_in7 : IN STD_LOGIC_VECTOR(7 DOWNTO 0);

m_clk : IN STD_LOGIC;

gateway_out : OUT STD_LOGIC_VECTOR(7 DOWNTO 0)

);

end mux;

architecture Behavioral of mux is

signal gzero : std_logic_vector(7 downto 0) := (others=>'0');

BEGIN

mux_proc: PROCESS(m_clk)

BEGIN

if(rising_edge(m_clk)) then

CASE data_in IS

WHEN"000"=>gateway_out<=gateway_in0;

WHEN"001"=>gateway_out<=gateway_in1;

WHEN"010"=>gateway_out<=gateway_in2;

WHEN"011"=>gateway_out<=gateway_in3;

WHEN"100"=>gateway_out<=gateway_in4;

WHEN"101"=>gateway_out<=gateway_in5;

WHEN"110"=>gateway_out<=gateway_in6;

WHEN"111"=>gateway_out<=gateway_in7;

WHEN OTHERS=>

gateway_out<=gzero;

END CASE;

end if;

END PROCESS mux_proc;

end Behavioral;

C.3 Clock_Driver VHDL code

----------------------------------------------------------------------------------------------------------------------------

-- Company: ATU

-- Engineer:  Ahmed M.  Alfadhel

-- Create Date: 06/25/2019 10:22:13 AM

-- Module Name: driver - Behavioral

-- Revision 0.01 - File Created

-- Additional Comments: This circuit is used to convert high freq. (5 MHz) to a low frequency (30 kHz).

----------------------------------------------------------------------------------------------------------------------------

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.numeric_std.all;

entity driver_v3 is

Port ( in_clk : in STD_LOGIC;

out_clk : out STD_LOGIC);

end driver_v3;

architecture Behavioral of driver_v3 is

constant maxcount  : integer := 85;

signal i_clk : std_logic := '0';

begin

out_clk <= i_clk;

driver_proc: process(in_clk)

variable i_en    : std_logic := '0';

variable counter :unsigned(6 downto 0) := to_unsigned(0, 7);

begin

if(rising_edge(in_clk)) then

if(i_en = '1')  then

i_clk <= not i_clk;

counter := (others => '0');

i_en := '0';

elsif (counter /=(maxcount - 1)) then

counter := counter +1;

else

i_en := '1';

end if;

end if;

end process driver_proc;

end Behavioral;

C.4 Abs VHDL code

----------------------------------------------------------------------------------------------------------------------------

-- Company: ATU

-- Engineer: Ahmed M. Alfadhel

-- Create Date: 07/27/2019 05:49:20 PM

-- Module Name: absolute - Behavioral

-- Revision 0.01 - File Created

-- Additional Comments: Convert a negative signed value to a positive signed value

----------------------------------------------------------------------------------------------------------------------------

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.math_real.all;

use IEEE.numeric_std.all;

entity absolute_16bits is

Port ( inp : in STD_LOGIC_VECTOR (15 downto 0);

outp : out STD_LOGIC_VECTOR (15 downto 0);

clk : in STD_LOGIC);

end absolute_16bits;

architecture Behavioral of absolute_16bits is

signal Xout_abs : signed(15 downto 0) := (others => '0');

signal Xin_abs : signed(15 downto 0) := (others => '0');

begin

Xin_abs <= signed(inp);                         -- we are performing a conversion from std_logic_vector to signed

outp <= std_logic_vector(Xout_abs);  -- we are performing a conversion from signed to std_logic_vector

abs_proc: process(clk)

begin

if(rising_edge(clk)) then

if (Xin_abs < 0) then

Xout_abs <= 0 - Xin_abs;

else

Xout_abs <= Xin_abs;

end if;

end if;

end process abs_proc;

end Behavioral;

C.5 RD_EN VHDL code

---------------------------------------------------------------------------------------------------------------------------

-- Company: ATU

-- Engineer: Ahmed M. Alfadhel

-- Create Date: 08/09/2019 12:01:59 PM

-- Module Name: rd_clk - Behavioral

-- Revision 0.01 - File Created

-- Additional Comments: This IP is providing read enable signal to FIFO genrator

---------------------------------------------------------------------------------------------------------------------------

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.numeric_std.all;

entity rd_clk is

Port ( in_clk : in STD_LOGIC;

out_en: out STD_LOGIC);

end rd_clk;

architecture Behavioral of rd_clk is

constant maxcount  : integer := 45;

-- wait for 45 clocks of 80 kHz. FIFO should always have 45 samples in it

signal i_read : std_logic := '0';

begin

out_en <= i_read;

driver_proc: process(in_clk)

variable i_en    : std_logic := '0';

variable counter :unsigned(6 downto 0) := to_unsigned(0, 7);

begin

if(rising_edge(in_clk)) then

if(i_en = '1')  then

i_read <= '1';

counter := (others => '0');

elsif (counter /=(maxcount - 1)) then

counter := counter +1;

else

i_en := '1';

end if;

end if;

end process driver_proc;

end Behavioral;

C.6 BFSK_demodulator VHDL code

---------------------------------------------------------------------------------------------------------------------------

-- Company: ATU

-- Engineer: Ahmed M. Alfadhel

-- Create Date: 08/06/2019 11:41:17 AM

-- Module Name: BFSK_demodulation3 - Behavioral

-- Revision 0.01 - File Created

-- Additional Comments: BFSK demodulator based on ZCD method.

---------------------------------------------------------------------------------------------------------------------------

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.numeric_std.all;

entity BFSK_demod4 is

Port ( clk : in STD_LOGIC;

Envelop_in : in STD_LOGIC_VECTOR (47 downto 0);

Recovered_Data : out STD_LOGIC);

end BFSK_demod4;

architecture Behavioral of BFSK_demod4 is

signal data : std_logic;

constant threshold: unsigned(47 downto 0):= “000000000010111101111111010110001111100000000000”   --204 Giga amplitude

constant maxcount  : integer := 7614;      -- 1500 usec

constant maxcountm  : integer := 508;   --100 usec

constant testing_point  : integer := 10;

begin

Recovered_data <= data;

FSK_dem_proc: process(clk)

variable i_en    : std_logic := '0';

variable counter :unsigned(16 downto 0) := to_unsigned(0, 17);

variable counter2 :unsigned(16 downto 0) := to_unsigned(0, 17);

variable counter3 :unsigned(16 downto 0) := to_unsigned(0, 17);

variable counter4 :unsigned(16 downto 0) := to_unsigned(0, 17);

begin

if(rising_edge(clk)) then

if(i_en = '1')  then

counter2 := counter2 +1;

if (counter2 = maxcountm) then

counter2 := (others => '0');

counter3 := counter3 + 1;

if (unsigned(Envelop_in) <=  threshold) then

counter4 := counter4 + 1;

end if;

if (counter3 = testing_point) then

counter3 := (others => '0');

if(counter4 <= 2) then

data <= '0';

counter4 := (others => '0');

else

data <= '1';

counter4 := (others => '0');

end if;

end if;

end if;

elsif(counter /=(maxcount - 1)) then

counter := counter +1;

else

i_en := '1';

end if;

end if;

end process FSK_dem_proc;

end Behavioral;

C.7 Test Bench VHDL code

---------------------------------------------------------------------------------------------------------------------------

-- Company: ATU

-- Engineer: Ahmed M. Alfadhel

-- Create Date: 27/03/2018 07:52:12 PM

-- Module Name: Test bench- Behavioral

-- Revision 0.01 - File Created

-- Additional Comments: This file is Used with Vivado IDE for Simulating the System design

---------------------------------------------------------------------------------------------------------------------------

-- Libraries

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.numeric_std.all;

use std.textio.all ;

use ieee.std_logic_textio.all ;

-- Entity

entity FHSS_TX_Test_Bench_sim is

end;

-- Architecture

architecture test of FHSS_TX_Test_Bench_sim is

-- Our Transmitter Design Instantiation

component FH_modem_wrapper

port (

BFSK : out STD_LOGIC_VECTOR ( 7 downto 0 );

FH : out STD_LOGIC_VECTOR ( 7 downto 0 );

spreaded_signal : out STD_LOGIC_VECTOR ( 7 downto 0 );

despreaded : out STD_LOGIC_VECTOR ( 7 downto 0 );

IF_BPF : out STD_LOGIC_VECTOR (15 downto 0);

HT : out STD_LOGIC_VECTOR ( 39 downto 0 );

delayed_IF : out STD_LOGIC_VECTOR ( 15 downto 0 );

absolute : out STD_LOGIC_VECTOR ( 39 downto 0 );

absolute2 : out STD_LOGIC_VECTOR ( 15 downto 0 );

LPF : out STD_LOGIC_VECTOR (47 downto 0);

LPF2 : out STD_LOGIC_VECTOR (23 downto 0);

adder_out : out STD_LOGIC_VECTOR ( 47 downto 0 );

Data_TX : out std_logic;

Recov_Data : out STD_LOGIC;

sys_clock : in STD_LOGIC;

reset : in STD_LOGIC

);

end component;

-- Simulation signals

signal clk_sim 			: std_logic := '0';

signal reset         	               : std_logic := '1';

signal Recovered_Data                 : std_logic := '0';

signal Data_TX           		 : std_logic := '0';

signal BFSK 			: std_logic_vector(7 downto 0);

signal FH 			: std_logic_vector(7 downto 0);

signal spreaded_signal 			: std_logic_vector(7 downto 0);

signal despreaded 			: std_logic_vector(7 downto 0);

signal IF_BPF              		              : std_logic_vector(15 downto 0);

signal HT              		              : std_logic_vector(39 downto 0);

signal adder_out             		              : std_logic_vector(47 downto 0);

signal absolute        			: STD_LOGIC_VECTOR ( 39 downto 0 );

signal delayed_IF      			  : STD_LOGIC_VECTOR ( 15 downto 0 );

signal absolute2      			  : STD_LOGIC_VECTOR ( 15 downto 0 );

signal LPF      				  : STD_LOGIC_VECTOR ( 47 downto 0 );

signal LPF2     				   : STD_LOGIC_VECTOR ( 23 downto 0 );

begin

-- Transmitter port mapping

dev_to_test:  FH_modem_wrapper

port map(BFSK, FH, spreaded_signal, despreaded, IF_BPF, HT ,delayed_IF, absolute, absolute2, LPF, LPF2,adder_out, Data_TX,Recovered_Data, clk_sim, reset );

-- Simulate the input clock to our design

clk_proc : process

begin

wait for 5 ns;

clk_sim <= not clk_sim;

end process clk_proc;

end test;