Parsing Data

The micropyGPS library is utilized to parse raw NMEA data from the GP-02 module, enabling rapid development and application of the module.

Code:

Name: GPS data translation

Version: v1.0

Date: 2025.05

Author: ljl

Other: Translate origin GPS data by micropyGPS library and print them in Shell

'''

from machine import UART,Pinfrom micropyGPS import MicropyGPSimport time

# initialize UART configuration

gps_uart = UART(0, baudrate=9600, tx=Pin(0), rx=Pin(1))

# initialize GPS translator

my_gps = MicropyGPS(8)  # east eight time zone

def update_gps():

""" Read GPS module """

while gps_uart.any():

char = gps_uart.read(1)

if char:

my_gps.update(chr(char[0]))

def print_gps_data():

""" print GPS data """

if my_gps.date and my_gps.timestamp:

date_str = f"{my_gps.date[2]}-{my_gps.date[1]:02d}-{my_gps.date[0]:02d}"

time_str = f"{my_gps.timestamp[0]:02d}:{my_gps.timestamp[1]:02d}:{int(my_gps.timestamp[2]):02d}"

latitude_str = f"{my_gps.latitude[0]:.6f} {my_gps.latitude[1]}"

longitude_str = f"{my_gps.longitude[0]:.6f} {my_gps.longitude[1]}"

speed_str = f"{my_gps.speed[2]:.2f} km/h"

print(f"Date: {date_str}")

print(f"Time: {time_str}")

print(f"Latitude: {latitude_str}")

print(f"Longitude: {longitude_str}")

print(f"Speed: {speed_str}")

else:

print("Waiting for GPS data...")

while True:

update_gps()

print_gps_data()

time.sleep(1)​

Once the code is executed, the terminal will display GPS - related information, including date, time, latitude, longitude, and speed.