If you are using VirtualBox or VMWare or a Linux system, log in to the system accordingly.

Step 1: You can create a new folder and enter it using the following command.

mkdir Anomaly_Detection;

cd Anomaly_Detection;

Step 2: Create a configurations file for this project, using the following command.

nano conf.py

After the editor is open, type in the following configuration parameters

SSID = 'You can find SSID in your Twilio Dashboard'

AUTH_TOKEN = 'You can find  on your Twilio Dashboard'

FROM_NUMBER = 'This is the no. generated by Twilio. You can find this on your Twilio Dashboard'

TO_NUMBER = 'This is your number. Make sure you are adding +91 in beginning'

API_KEY = 'This is your Bolt Cloud account API key'

DEVICE_ID = 'This is the ID of your Bolt device'

FRAME_SIZE = 10

MUL_FACTOR = 6

Note: You have to replace all the above value with your credentials. You can find the first four values in Twiliodashboard and the last two in Bolt Cloud dashboard.

You can set the FRAME_SIZE to 10, and the MUL_FACTOR to 3 for now. Once done, you can save the configurations file by pressing 'CTRL+x'

Step 3: Now create one more file named anomaly_detction.py, using the following command

sudo nano anomaly_detection.py

This file will contain the main code. The algorithm for the code can be broken down into the following steps:

1) Fetch the latest sensor value from the Bolt device.

2) Store the sensor value in a list, that will be used for computing z-score.

3) Compute the z-score and upper and lower threshold bounds for normal and anomalous readings.

4) Check if the sensor reading is within the range for normal readings.

5) If it is not in range, send the SMS.

6) Wait for 10 seconds.

7) Repeat from step 1.

Step 6: Here we will start writing the code. Let us start with the imports.

import conf, json, time, math, statistics

from boltiot import Sms, Bolt

We have already seen why we need to import conf, json, time, Sms and Bolt in the earlier lessons. The math and statistics libraries will be required for calculating the Z-score and the threshold boundaries.

The following lies code helps define a function which calculates the Z-score and the using the Z-score calculates the boundaries required for anomaly detection.

def compute_bounds(history_data,frame_size,factor):

The above line helps define a function, which takes 3 input variables: hisotry_data, frame_size and factor.

if len(history_data)<frame_size :

return None

if len(history_data)>frame_size :

del history_data[0:len(history_data)-frame_size]

The above code checks whether enough data has been accumulated to calculate the Z-score, and if there is too much data, then the code deletes the older data.

Mn=statistics.mean(history_data)

The above code calculates the mean (Mn) value of the collected data points.

Variance=0

for data in history_data :

Variance += math.pow((data-Mn),2)

This code helps to calculate the Variance of the data points. You can read more about variance here.

Zn = factor * math.sqrt(Variance / frame_size)

High_bound = history_data[frame_size-1]+Zn

Low_Bound = history_data[frame_size-1]-Zn

return [High_bound,Low_Bound]

Here we calculate the Z score (Zn) for the data and use it to calculate the upper and lower threshold bounds required to check if a new data point is normal or anomalous.

The next code is used to initialize the Bolt and SMS variables, which we will use to collect data and send SMS alerts. Here we also initialize an empty list with the name 'history_data' which we will use to store older data, so that we can calculate the Z-score.

mybolt = Bolt(conf.API_KEY, conf.DEVICE_ID)

sms = Sms(conf.SSID, conf.AUTH_TOKEN, conf.TO_NUMBER, conf.FROM_NUMBER)

history_data=[]

The following while loop contains the code required to run the algorithm of anomaly detection.

while True:

response = mybolt.analogRead('A0')

data = json.loads(response)

if data['success'] != '1':

print("There was an error while retriving the data.")

print("This is the error:"+data['value'])

time.sleep(10)

continue

print ("This is the value "+data['value'])

sensor_value=0

try:

sensor_value = int(data['value'])

except Exception as e:
        print("There was an error while parsing the response: ",e)

continue

bound = compute_bounds(history_data,conf.FRAME_SIZE,conf.MUL_FACTOR)

if not bound:

required_data_count=conf.FRAME_SIZE-len(history_data)

print("Not enough data to compute Z-score. Need ",required_data_count," more data points")

history_data.append(int(data['value']))

time.sleep(10)

continue

try:

if sensor_value > bound[0] :

print ("The light level increased suddenly. Sending an SMS")

response = sms.send_sms("Someone turned on the lights")

print("This is the response ",response)

elif sensor_value < bound[1]:

print ("The light level decreased suddenly. Sending an SMS")

response = sms.send_sms("Someone turned off the lights")

print("This is the response ",response)

history_data.append(sensor_value);

except Exception as e:

print ("Error",e)

time.sleep(10)

Here is the full code in one frame, you can copy and paste this code into your anomaly_detection.py file.

import conf, json, time, math, statistics

from boltiot import Sms, Bolt

def compute_bounds(history_data,frame_size,factor):

if len(history_data)<frame_size :

return None

if len(history_data)>frame_size :

del history_data[0:len(history_data)-frame_size]

Mn=statistics.mean(history_data)

Variance=0

for data in history_data :

Variance += math.pow((data-Mn),2)

Zn = factor * math.sqrt(Variance / frame_size)

High_bound = history_data[frame_size-1]+Zn

Low_Bound = history_data[frame_size-1]-Zn

return [High_bound,Low_Bound]

mybolt = Bolt(conf.API_KEY, conf.DEVICE_ID)

sms = Sms(conf.SSID, conf.AUTH_TOKEN, conf.TO_NUMBER, conf.FROM_NUMBER)

history_data=[]

while True:

response = mybolt.analogRead('A0')

data = json.loads(response)

if data['success'] != '1':

print("There was an error while retriving the data.")

print("This is the error:"+data['value'])

time.sleep(10)

continue

print ("This is the value "+data['value'])

sensor_value=0

try:

sensor_value = int(data['value'])

except Exception as e:
        print("There was an error while parsing the response: ",e)

continue

bound = compute_bounds(history_data,conf.FRAME_SIZE,conf.MUL_FACTOR)

if not bound:

required_data_count=conf.FRAME_SIZE-len(history_data)

print("Not enough data to compute Z-score. Need ",required_data_count," more data points")

history_data.append(int(data['value']))

time.sleep(10)

continue

try:

if sensor_value > bound[0] :

print ("The light level increased suddenly. Sending an SMS.")

response = sms.send_sms("SoMe one has opened the door…!")

print(response)

elif sensor_value < bound[1]:

print ("The light level decreased suddenly. Sending an SMS.")

response = sms.send_sms("Hey..control the Temperature.")

print(response)

history_data.append(sensor_value);

except Exception as e:

print ("Error",e)

time.sleep(10)

Step 4: Save the file by pressing Ctrl+x.