SCHOOL OF COMPUTING (SOC)

| Date of Submission: | 17th February 2019 |
| --- | --- |

| Prepared for: | Dora Chua |
| --- | --- |

| Class: | DIT/FT/2A/33 |
| --- | --- |

| Submitted by: |  |
| --- | --- |

| Student ID | Name |
| --- | --- |
| 1727526 1727740 1727836 | Jasmin Yap Yi Taperla Anne Muriele Tatac Kevan Ong Jing Jie |

IOT CA2

Step-by-step Tutorial

DIPLOMA IN INFORMATION TECHNOLOGY

ST0324 Internet of Things (IOT)

2018/2019 Semester 2

- Table of Contents

Section 1 Overview of the application	4

A.	Where have we uploaded our tutorial?	4

B.	Why have we chosen to upload to this site?	4

C.	What have we uploaded?	4

D.	What is the application about?	4

E.	Summary of steps	4

F.	What does the final setup look like?	5

Room One	6

Room Two	7

G.	How does the web application look like?	7

login.html	8

Index.html	8

RoomOne.html	9

RoomTwo.html	9

H.	How does the final Fritzing diagram look like?	10

Room One	10

Room Two	11

Section 2 Hardware requirements	12

Hardware checklist	12

DHT11 Temperature and Humidity Sensor	12

Resistors (10K Ω and 220/330 Ω)	12

LED	13

Buzzer	13

RFID/NFC MFRC522 Card Reader Module	13

PIR Motion Sensor	14

Section 3 Setting up the database using command-line	14

A.	Run mysql command and create database	14

B.	Create rootadmin to access assignmentDB	15

C.	Login as rootadmin and create table	15

D.	Insert data into UID and login table	16

Section 4 Create IoT “Thing”	17

A.	Sign into the AWS IoT Console	17

B.	Create and register your “Thing”	17

C.	Create a policy for your Thing	20

D.	Attach Security Policy and Thing to your Certificate	20

E.	Copy the REST API endpoint of your “Thing”	22

Section 5 Install the AWS Python SDK	23

A.	Install the AWS Python Library	23

Section 6 Create tables in DynamoDB	24

A.	Access DynamoDB	24

B.	Create RBM_temp	24

C.	Create RBM_rfidRecords	25

Section 7 Setting up AWS rules	28

A.	Create Rule for Temperature and Humidity Topic	28

B.	Create Rule for RFID Topic	31

Section 8 Setting up DHT11 (Room One & Two)	34

A.	Frizting diagram (DHT11 ONLY)	34

B.	Connecting the components	34

C.	dht.py	34

Section 9 Setting up RFID/NFC reader (Room One & Two)	37

A.	Fritzing Diagram (MFRC522 ONLY)	37

B.	Connecting the components	37

C.	Enable SPI and Prepare the MFRC522 libraries	38

D.	read_display.py	39

Section 10 Setting up Buzzer (Room One & Two)	42

A.	Fritzing Diagram (Buzzer ONLY)	42

B.	Connecting the components	42

Section 11 Setting up LED (Room One ONLY)	43

A.	Fritzing Diagram (LED ONLY)	43

B.	Connect the components	43

C.	lights.py	43

Section 12 Setting up Motion Sensor Light (Room Two Only)	45

A.	Fritzing Diagram (Motion sensor & LED ONLY)	45

B.	Connecting the components	45

C.	Pushover	45

D.	testt.py	47

Section 13 Code for Web Application	49

A.	Room One fritzing diagram	49

B.	Room Two fritzing diagram	49

C.	server.py	50

D.	dynamoDB.py	54

E.	jsonconverter.py	56

F.	Templates	57

Index.html	58

login.html	64

RoomOne.html	66

RoomTwo.html	73

E.	Run the program 	79

# Section 1
Overview of the application

  - Where have we uploaded our tutorial?

https://www.hackster.io/NMuriele/random-security-company-101-997b28

  - Why have we chosen to upload to this site?

We have chosen to upload our project on Hackster as it is the world’s fastest growing developer community for learning and building hardware. They are notable for their neat navigation system, and the projects are filtered according to the hardware platform you wish to use or learn. This will help the users save time learning how to navigate the site as it is already organised. Apart from this, Raspberry Pi projects are in abundance in Hackster, making it a sensible choice.

  - What have we uploaded?

We have uploaded the step-by-step tutorial, fritzing diagram and the full source code.

  - What is the application about?

This application is meant to monitor two temperature and humidity sensitive rooms where authorised staff need to scan a card to enter. A web application will display the temperature and humidity data collected from the room in a table and graph. The card ID as well as the date and time of successful entries will also be displayed in a table.

As mentioned above, both rooms require the staff to scan their cards upon entry. An authorised card will make one short buzz to simulate a successful entry. Meanwhile, unauthorised cards, will make two short buzzes to simulate a failed entry.

Room One has LED light that can be controlled through the web application. On the other hand, Room Two has motion sensor controlled LED light. When motion is detected in the room, the light turns on and a push notification is sent to the admin’s smartphone. This is achieved using Pushover.

  - Summary of steps

|  | Section | Description |
| --- | --- | --- |
|  | Overview | What the project is about and what it will look like. |
|  | Hardware requirements | Provides overview of hardware required. |
|  | Setting up the database | Provides code to set up the required database and tables. |
|  | Create IoT “Thing” | We will sign into Amazon Web Services and set up the necessary items for the programme to run. |
|  | Install the AWS Python SDK | Before you can start coding an IoT app on the AWS Cloud, you will need to install the AWS Python software libraries first. |
|  | Create tables in DynamoDB | We will create the necessary tables in DynamoDB for the programme to run and be accessed from cloud. |
|  | Setting up AWS rules | We will create and configure a rule to send the data received from a device to DynamoDB |
| Section 8-13 will show you how to set up the application |  |  |
|  | Setting up DHT11 | Connect the hardware components and write python code for DHT11. |
|  | Setting up RFID/NFC Reader | Connect the hardware components for RFID/NFC reader. Code to read the RFID is also included. |
|  | Setting up Buzzer | Connect the hardware components for Buzzer and write python code for Buzzer. |
|  | Setting up LED | Connect the hardware components for LED. |
|  | Setting up Motion Sensor Light | Connect the hardware components for Motion Sensor and write python code for Motion Sensor. |
|  | Code for Web Application | Writing the code for server and front end of the web application. |

Note: we will be doing the connection part by part, therefore please do not remove any connections done when moving on to the next section

  - What does the final setup look like?

### Room One

### Room Two

  - How does the web application look like?

### login.html

### Index.html

### RoomOne.html

### RoomTwo.html

  - How does the final Fritzing diagram look like?

### Room One

### Room Two

# Section 2
Hardware requirements

- Hardware checklist

### DHT11 Temperature and Humidity Sensor

| - Low-cost digital temperature and humidity sensor - Uses capacitive humidity sensor and a thermistor to measure the surrounding air - Produces digital signal on the data pin, no analog input pins needed - Less accurate and has a smaller range compared to the DHT22 but is small and less expensive - Good for 20-80% humidity readings with 5% accuracy - Good for 0-50°C temperature readings ±2°C accuracy All information above provided by adafruit.com | All images credited to adafruit.com |
| --- | --- |

### Resistors (10K Ω and 220/330 Ω)

| - Carbon film - Through-hole axial type - 1/4 Watt max dissipation - 300V max voltage - 5% precision resistors All information above provided by adafruit.com For DHT11 Sensor: 10K Ω Due to the data line being bidirectional, a pullup resistor is needed to connect the data line and VCC line of the sensor. For LED: 220 Ω or 330 Ω Resistor is always required for LEDs as they will draw more power and cause the Raspberry Pi to burn out. Note: I use 220 Ω, 330 Ω looks similar to 220 Ω just that the first two bands are orange instead of red | 10k Ω 220 Ω All images credited to adafruit.com |
| --- | --- |

### LED

| - 5mm diameter - 660 nm wavelength - 1.85-2.5V Forward Voltage, at 20mA current - 250 mcd typical brightness All information above provided by adafruit.com Things to note: Longer leg anode (power) Shoerter leg cathode (ground) | All images credited to adafruit.com |
| --- | --- |

### Buzzer

| - An audio signaling device found in circuits to create a beeping noise. - Active buzzers and passive ones. For our lab, we will use active buzzers as they are a lot simpler to use. - Do not need a resistor - Typically has 2 pins: - VOUT – Connect this to a GPIO pin to control its value - GND – Connect this to ground |  |
| --- | --- |

### RFID/NFC MFRC522 Card Reader Module

| - Contactless reader module - Low power consumption - Small and low cost All information above provided by sgbotic.com Radio-frequency identification (RFID) uses electromagnetic fields to identify and track objects that have been tagged. Examples of uses include EZlink cards, contactless credit cards etc. | All images credited to hobbytronics.co.uk |
| --- | --- |

### PIR Motion Sensor

| - Sensors nearby movement - The sensor detects the pattern of infrared energy in its surroundings. If the pattern changes, the sensor outputs a high signal. - has a trimpot on its back for adjusting its time delay and sensitivity - The sensor has 3 pins. - VCC – Connect this to power (5V or 3.3V) - VOUT – Connect this to a GPIO pin to read its value - GND – Connect this to ground |  |
| --- | --- |

# Section 3
Setting up the database using command-line

Before setting up the breadboard, we will have to set up the database that stores the login values and the RFID card ID values that will be used in the project.

- Run mysql command and create database

|  | Open the Terminal window and run the following command: mysql -uroot -pdmitiot |
| --- | --- |
|  | Once you have connected to MySQL, you can create the database using the following command: mysql> create database assignmentDB; |
|  | You should see the following message if you are successful: Query Ok, 1 row affected (0.00 sec) |

- Create rootadmin to access assignmentDB

|  | Use the following command to use the database that you created earlier: mysql> USE assignmentDB |
| --- | --- |
|  | Create a user with the password “root” that has access to the database mysql> CREATE USER ‘rootadmin’@’localhost’ IDENTIFIED by ‘root’; mysql> GRANT ALL PRIVILEGES ON assignmentDB.* TO ‘rootadmin’@’localhost’; mysql> FLUSH PRIVILEGES; mysql> quit |
|  | You should see the following message if you are successful: Query Ok, 1 row affected (0.00 sec) |
|  | Exit the MySQL prompt using Ctrl+D and close the Terminal window |

- Login as rootadmin and create table

|  | Open a Terminal window and type the following command to log in mySQL. When prompted with the password, type in the password “root” mysql -urootadmin -p |
| --- | --- |
|  | Use the following command to use the database that you created earlier: mysql> USE assignmentDB |
|  | Type in the following code to create a table named login that has 4 fields, id, Name, Username and Password of type int, varchar(100) and varchar(30) for the last two. mysql> CREATE TABLE dht (id int NOT NULL AUTO_INCREMENT, Username varchar(30), Password varchar(30), PRIMARY KEY (id)); |
|  | You should see the following message if you are successful: Query Ok, 1 row affected (0.00 sec) |
|  | Create another table to store the RFID values names UID that has 2 fields, ID and Admin of type varchar(25) and varchar(3). mysql> CREATE TABLE UID (ID varchar(25) NOT NULL, Admin varchar(3), PRIMARY KEY (ID)); |
|  | You should see the following message if you are successful: Query Ok, 1 row affected (0.00 sec) |

- Insert data into UID and login table

In order for the authentication process to work, at least two rows of data (one row admin and one not) needs to be in the UID table and one row of data must be added into the login table before running the project.

|  | Get the card ID of two RFID cards and store them in the database. mysql> INSERT into UID (ID, Admin) VALUES (RFIDno, ‘yes’), (RFIDno, ‘no’); EXAMPLE: Replace ‘RFIDno’ with the card ID that you have, eg. [136, 4, 0, 75, 199] and [136, 4, 16, 170, 54] are the card IDs that I have, thus the code looks like this: mysql> INSERT into UID (ID, Admin) VALUES (‘[136, 4, 0, 75, 199]’, ‘yes’), (‘[136, 4, 16, 170, 54]’, ‘no’); |
| --- | --- |
|  | You should see the following message if you are successful: Query Ok, 1 row affected (0.00 sec) Records: 2 Duplicates: 0 Warnings: 0 |
|  | Come up with a username and a password for logging into the web application and add it to the database using this command mysql> INSERT into login (Name, Username, Password) VALUES (‘INSERT USERNAME’, ‘INSERT PASSWORD’); |
|  | You should see the following message if you are successful: Query Ok, 1 row affected (0.00 sec) Records: 1 Duplicates: 0 Warnings: 0 |
|  | Exit the MySQL prompt using Ctrl+D and close the Terminal window |

# Section 4
Create IoT “Thing”

Now,  we will sign into Amazon Web Services and set up the necessary items for the programme to run.

- Sign into the AWS IoT Console

Assuming that you already have a functioning AWS account that can be used.

|  | Sign in with your AWS console at https://aws.amazon.com |
| --- | --- |
|  | In the AWS dashboard, search for IoT Core. |
|  | On the Welcome page, choose Get Started. |

- Create and register your “Thing”

|  | In the navigation pane on the left, choose Manage |  |
| --- | --- | --- |
|  | On the page, (if you have not made a thing before) it should say “You don’t have any things yet”, choose “Register a thing”. If you have created a thing before, choose Create. A thing here represents a device whose status or data is stored in the AWS cloud. The Thing Shadows is the stats of the device, e.g is it “on” or “off” etc Our Thing will be the RPi that is used to host our server. |  |
|  | Select “Create a single thing” |  |
|  | Give your thing a name. |  |
|  | Select “Create certificate” |  |
|  | Create a folder on your Raspberry Pi named “RSC” and download the certificates to that folder. Note: The file you download this to must be in the same file as all of your code. For the root CA download, right click the first one and select “save link as”: Afterwards, be sure to click the activate button before clicking done. |  |

- Create a policy for your Thing

|  | In the navigation pane on the left, choose Secure and Policies Once in the tab, click “Create”. |  |
| --- | --- | --- |
|  | On the Create a policy page, key in the following configuration: Field; Type this in. Name; MyRaspberryPiSecurityPolicy. Action; iot:*. Resource ARN; *. Allow; Checked. |
|  | Click “Create” You now have a Ssecurity Policy that allows all access to IoT Core services. |  |

- Attach Security Policy and Thing to your Certificate

In this section, you will attach both your security policy and your Thing to your certficate that you created previously.

|  | In the navigation pane on the left, choose Secure and Policies. |  |
| --- | --- | --- |
|  | Find the Certificate starting with the same string as the one you created and downloaded earlier. When I created the certificate earlier, it started with “b4cf9a654c”. So I will select the certificate starting with the same string and click “attach policy” under the “Actions” button. |  |
|  | Select “MyRaspberryPiSecurityPolicy” that was created earlier and click “Attach” |  |
|  | When I created the certificate earlier, it started with “b4cf9a654c”. So I will select the certificate starting with the same string and click “attach Thing” under the “Actions” button. |  |
|  | Find the Thing you created earlier and select it. |  |

- Copy the REST API endpoint of your “Thing”

|  | Click “Manage->Things” and choose “MyRaspberryPi”. On the next screen, choose “Interact” |  |
| --- | --- | --- |
|  | Copy and paste the REST API endpoint into a Notepad. You will need this value later. |  |

# Section 5
Install the AWS Python SDK

By now, you have  completed the first part of the process to connect your device to AWS cloud.

In the next few sections, you will begin to code your app. However, before you can start coding an IoT app on the AWS Cloud, you will need to install the AWS Python software libraries first.

- Install the AWS Python Library

|  | Install the AWS Python library with this command sudo pip install AWSIoTPythonSDK |
| --- | --- |

# Section 6
Create tables in DynamoDB

Here, we will create the necessary tables in DynamoDB for the programme to run.

- Access DynamoDB

|  | Navigate to the AWS console and search for DynamoDB. Select DynamoDB |
| --- | --- |
| 2) | Select Tables from this page |

- Create RBM_temp

| 1) | Next, click on create table to create a table |
| --- | --- |
| 2) | Create the table with the following details Field; Type this in. Table Name; RBM_temp. Primary Key. Partition Key; deviceid. Sort Key; datetimeid. |
| 3) | You should end with this table. |

- Create RBM_rfidRecords

| 1) | Next, click on create table to create a table |
| --- | --- |
| 2) | Create the table with the following details Field; Type this in. Table Name; RBM_rfidRecords. Primary Key. Partition Key; deviceid. Sort Key; datetime. RBM_rfidRecords |
| 3) | You should end with this table. RBM_rfidRecords |

# Section 7
Setting up AWS rules

The AWS IoT rules engine listens for incoming MQTT messages that match a rule. When a matching message is received, the rule takes some action with the data in the MQTT message.  In this step, you will create and configure a rule to send the data received from a device to DynamoDB.

- Create Rule for Temperature and Humidity Topic

|  | In the AWS IoT console, in the left navigation pane, choose Act, then “Create a rule” and enter the following data. |
| --- | --- |
| - Ss | Select Add action |
|  | Select the second option ‘Split message into multiple columns of a DynamoDB table (DynamoDBv2) |
|  | Click on configure action and enter the following data. Once you’ve entered the data, click “add action” |
|  | This should look like what it’ll look like |

- Create Rule for RFID Topic

|  | In the AWS IoT console, in the left navigation pane, choose Act, then “Create a rule” and enter the following data. |  |
| --- | --- | --- |
|  | Select an action |  |
|  | Select the second option ‘Split message into multiple columns of a DynamoDB table (DynamoDBv2) |  |
|  | Click on configure action and enter the following data. RBM_rfidRecords Click “add action” once you’re done. |  |
|  | This is what its suppose to look like |  |

# Section 8
Setting up DHT11 (Room One & Two)

In this section we will be setting up the DHT11 Sensor as well as the code needed to run it.

The DHT11 sensor is needed in order to gather the data that is displayed on the web application.

- Frizting diagram (DHT11 ONLY)

GROUND RAIL

POWER

GPIO 4

POWER

10k Ω RESISTOR

GROUND

- Connecting the components

|  | Plug the DHT11 sensor into the breadboard at the very end. This is to ensure that it does not interfere with the other components that will be plugged in later. |
| --- | --- |
|  | Using a red wire, connect 3.3v to the power rail as shown. Using a black wire, connect the ground pin to the power rail on the oppsite side as shown. This will be refered to as the ground rail. |
|  | The DHT11 sensor has 4 pins. Add a 10k Ω resistor to the VCC and DATA pin. Simply connect the pins as shown starting from the left-most pin (closest to the T-Cobbler Kit): DHT11 Sensor; Raspberry Pi; Jumper Colour. VCC; Power Rail; Red. DATA; GPIO 4; Blue. NC. GND; Ground Rail; Black. |

- dht.py

Now, we shall write a Python script that will read the current temperature and humidity values from the sensor and store them in the database table “dht” that was created earlier.

|  | Change the directory to RSC, the same file your certificates are in cd ~/RSC |
| --- | --- |
|  | Create a python script dht.py with the code below sudo nano ~/RSC/dht.py |
|  | Please take note of the indentations! Remember to replace the tokens with the ones that you have. # Import SDK packages from AWSIoTPythonSDK.MQTTLib import AWSIoTMQTTClient from time import sleep import Adafruit_DHT # Custom MQTT message callback def customCallback(client, userdata, message): print("Received a new message: ") print(message.payload) print("from topic: ") print(message.topic) print("--------------\n\n") # to write to AWS dynamoDB host = ""#Insert your host rootCAPath = "" #Insert your Amazon Root file certificatePath = "" #Insert your certificate file privateKeyPath = "" #Insert your private key file my_rpi = AWSIoTMQTTClient("RBMOnebasicPubSub") my_rpi.configureEndpoint(host, 8883) my_rpi.configureCredentials(rootCAPath, privateKeyPath, certificatePath) my_rpi.configureOfflinePublishQueueing(-1) # Infinite offline Publish queueing my_rpi.configureDrainingFrequency(2) # Draining: 2 Hz my_rpi.configureConnectDisconnectTimeout(10) # 10 sec my_rpi.configureMQTTOperationTimeout(5) # 5 sec # Connect and subscribe to AWS IoT my_rpi.connect() my_rpi.subscribe("RBM/sensors/temp", 1, customCallback) sleep(2) # Publish to the same topic in a loop forever loopCount = 0 while True: pin = 4 humidity, temperature = Adafruit_DHT.read_retry(11, pin) loopCount = loopCount+1 message = {} message["deviceid"] = "RBM_RoomOne" #add room number here import datetime as datetime now = datetime.datetime.now() message["datetimeid"] = now.isoformat() message["temp"] = temperature message["hum"] = humidity import json my_rpi.publish("RBM/sensors/temp", json.dumps(message), 1) sleep(5) |
|  | Run the program sudo python lights.py |
|  | Repeat the steps for the Raspberry Pi that is hosting Room 2 |

Note: we will be doing the connection part by part, therefore please do not remove any connections done when moving on to the next section

# Section 9
Setting up RFID/NFC reader (Room One & Two)

In this section, we will be setting up the MFRC522 card reader. This reader will be playing a part in the authentication that an authoriased staff is accessing the room.

- Fritzing Diagram (MFRC522 ONLY)

POWER

MFRC522

SDACEO

SCK SCLK

MOSI

MISO

GROUND RAIL

GPIO 25

- Connecting the components

The 9 pins on the MFRC522 have their respective pin names on the back.

Simply connect the pins accordingly:

| MFRC522 | Raspberry Pi | Jumper Colour |
| --- | --- | --- |
| SDA | CEO | Yellow |
| SCK | SCLK | Orange |
| MOSI | MOSI | Green |
| MISO | MISO | Blue |
| IDR |  |  |
| GND | Ground Rail | Black |
| RST | GPIO 25 | White |
| 3.3V | Power Rail | Red |
| 5V |  |  |

If the connection has been done correctly, a green light will be lit.

- Enable SPI and Prepare the MFRC522 libraries

If you have never worked with an MFRC522 card reader before, there are some steps that you need to take before the card reader can work. However, if you have worked with it before please skip this part.

|  | Run sudo raspi-config Choose the 5th menu item and enable SPI |
| --- | --- |
|  | Modify the /boot/config.txt to enable SPI sudo nano /boot/config.txt Ensure that the following lines are included in config.txt device_tree_param=spi=on dtoverlay=spi-bcm2835 |
|  | Install the Python development libraries sudo apt-get install python-dev |
|  | Set up the SPI Python libraries since the card reader uses the SPI interface cd ~ git clone https://github.com/lthiery/SPI-Py.git cd ~/SPI-Py sudo python setup.py install |
|  | Clone the MFRC522-python library to your home folder cd ~ git clone https://github.com/mxgxw/MFRC522-python.git |
|  | Copy the MFRC522.py into the folder that you are working in: sudo cp ~/MFRC522-python/*.py ~/RSC If you don’t do this, the reader will not work. |

- read_display.py

|  | Change the directory to RSC cd ~/RSC |
| --- | --- |
|  | Create a python script read_display.py with the code below sudo nano ~/RSC/read_display.py |
|  | Please take note of the indentations! Remember to replace the tokens with the ones that you have. import RPi.GPIO as GPIO import MFRC522 import signal import datetime from gpiozero import Buzzer from time import sleep import mysql.connector from AWSIoTPythonSDK.MQTTLib import AWSIoTMQTTClient #authorisation bz = Buzzer(5) uid = None continue_reading = True GPIO.setmode(GPIO.BCM) #connect to database mydb = mysql.connector.connect( host="localhost", user="rootadmin", passwd="root", database="assignmentDB" ) mycursor = mydb.cursor() # to write to AWS dynamoDB host = ""#Insert your host rootCAPath = "" #Insert your rootCAPath certificatePath = "" #Insert your certificatePath privateKeyPath = "" #Insert your privateKeyPath my_rpi = AWSIoTMQTTClient("RBMOnebasicPubSub") my_rpi.configureEndpoint(host, 8883) my_rpi.configureCredentials(rootCAPath, privateKeyPath, certificatePath) my_rpi.configureOfflinePublishQueueing(-1) # Infinite offline Publish queueing my_rpi.configureDrainingFrequency(2) # Draining: 2 Hz my_rpi.configureConnectDisconnectTimeout(10) # 10 sec my_rpi.configureMQTTOperationTimeout(5) # 5 sec # Connect and subscribe to AWS IoT my_rpi.connect() # for authorisation def checkUID(rfid): mycursor.execute("SELECT * FROM UID") myresult = list(mycursor.fetchall()) for x in myresult: if x[0] == rfid and x[1] == "yes": return True else: return False # Capture SIGINT for cleanup when the script is aborted def end_read(signal,frame): global continue_reading print("Ctrl+C captured, ending read.") continue_reading = False GPIO.cleanup() GPIO.setmode(GPIO.BCM) # end of authorisation functions # authorisation # Hook the SIGINT signal.signal(signal.SIGINT, end_read) # Create an object of the class MFRC522 mfrc522 = MFRC522.MFRC522() while continue_reading: # Scan for cards (status,TagType) = mfrc522.MFRC522_Request(mfrc522.PICC_REQIDL) # If a card is found if status == mfrc522.MI_OK: # Get the UID of the card (status,uid) = mfrc522.MFRC522_Anticoll() print(uid) #Checks if he is an authorised person if checkUID(str(uid)): bz.on() sleep(1) bz.off() message = {} message["deviceid"] = "RBM_RoomOne" #add room number import datetime as datetime now = datetime.datetime.now() message["datetime"] = now.isoformat() message["RFID"] = ("{}").format(uid) import json my_rpi.publish("RBM/sensors/rfid", json.dumps(message), 1) sleep(2) else: bz.on() sleep(0.5) bz.off() sleep(0.5) bz.on() sleep(0.5) bz.off() # currentDT = (datetime.datetime.now()).strftime("%Y-%m-%d %H:%M:%S") # filepath = ('/home/pi/CA2/pictures/{0}.jpg').format(currentDT) # camera.capture(filepath) # sleep(2) #end of authorisation |
|  | Run the program sudo python read_display.py |
|  | Repeat the steps for the Raspberry Pi that is hosting Room 2 |

Note: we will be doing the connection part by part, therefore please do not remove any connections done when moving on to the next section

# Section 10
Setting up Buzzer (Room One & Two)

In this section, we will be setting up the Buzzer that will signal whether the RFID card is accepted with one beep being the card accepted and two beeps being the card is rejected. The code is integrated with the code for the RFID.

- Fritzing Diagram (Buzzer ONLY)

- Connecting the components

Connect the Buzzer with the RPi as follows:

| Jumper color | PIR pin | RPi pin |
| --- | --- | --- |
| Black | GND | GND |
| Blue | VOUT | BCM5 |

Note: we will be doing the connection part by part, therefore please do not remove any connections done when moving on to the next section

# Section 11
Setting up LED (Room One ONLY)

In this section, we are setting up the MQTT controlled LED in Room One.

- Fritzing Diagram (LED ONLY)

- Connect the components

|  | Plug the LED into the breadboard. Place the long end nearer to the T-Cobbler Kit. |
| --- | --- |
|  | Plug in the 220 Ω resistor into the breadboard. One end of the resistor should be in the same column as the short leg of the LED. |
|  | Using a white cable, connect the long leg of the LED to GPIO 17. |
|  | Using a black cable, connect the free end of the resistor to the power rail that is connected to the ground (Ground rail). |

- lights.py

|  | Change the directory to RSC cd ~/RSC |
| --- | --- |
|  | Create a python script lights.py with the code below sudo nano ~/RSC/lights.py |
|  | Please take note of the indentations! Remember to replace the tokens with the ones that you have. from gpiozero import LED from AWSIoTPythonSDK.MQTTLib import AWSIoTMQTTClient from time import sleep import json light = LED(18) while True: try: # Custom MQTT message callback def customCallback(client, userdata, message): print("Received a new message: ") print(message.payload) print("from topic: ") print(message.topic) print("--------------\n\n") command = json.loads(message.payload) print(command["command"]) if command["command"] == "on": light.on() elif command["command"] == "off": light.off() # to write to AWS dynamoDB host = "" #Insert your host rootCAPath = "" #Insert your rootCAPath certificatePath = "" #Insert your certificationPath privateKeyPath = "" #Insert your privateKeyPath my_rpi = AWSIoTMQTTClient("RBMOnebasicPubSub") my_rpi.configureEndpoint(host, 8883) my_rpi.configureCredentials(rootCAPath, privateKeyPath, certificatePath) my_rpi.configureOfflinePublishQueueing(-1) # Infinite offline Publish queueing my_rpi.configureDrainingFrequency(2) # Draining: 2 Hz my_rpi.configureConnectDisconnectTimeout(10) # 10 sec my_rpi.configureMQTTOperationTimeout(5) # 5 sec # Connect and subscribe to AWS IoT my_rpi.connect() my_rpi.subscribe("RBM/LightSwitch", 1, customCallback) except: import sys print(sys.exc_info()) |
|  | Run the program sudo python lights.py |

Note: we will be doing the connection part by part, therefore please do not remove any connections done when moving on to the next section

# Section 12
Setting up Motion Sensor Light (Room Two Only)

In this section, we will be setting up how to connect the motion sensor to the raspberry pi.

- Fritzing Diagram (Motion sensor & LED ONLY)

- Connecting the components

Connect the PIR motion sensor with the RPi as follows:

| Jumper color | PIR pin | RPi pin |
| --- | --- | --- |
| Red | VCC | 3.3V |
| Black | GND | GND |
| Yellow | VOUT | BCM26 |

- Pushover

Here, we will configure Pushover which will send push notifications to your smartphone when motion is detected in the room.

|  | Go to http://pushover.net/ and sign up for an account. |
| --- | --- |
|  | Remember the user key provided as it will be used later. |
|  | Create an application. |
|  | Remember the API token that was generated when the application is successfully created as it will be used for later. |
|  | Install the Pushover application in your smartphone. https://pushover.net/clients Then, log in with the push over account you have created in Step a). Add your smartphone as one of the devices. |
|  | You can send a test notification to your smartphone device. |
|  | If the test notification is successful, your phone would show something like this: |

- testt.py

|  | Change the directory to RSC cd ~/RSC |
| --- | --- |
|  | Create a python script testt.py with the code below sudo nano ~/RSC/testt.py |
|  | Please take note of the indentations! Remember to replace the necessary tokens and strings with the ones you have. from gpiozero import LED, MotionSensor import time from time import sleep import httplib, urllib pir = MotionSensor(26, sample_rate=5,queue_len=1) light = LED(18) #Pushover API things PUSH_TOKEN = "" #Enter your app token PUSH_USER = "" #Enter your user token PUSH_MSG = "Motion is detected. Someone has entered the room." def sendPush( msg ): conn = httplib.HTTPSConnection("api.pushover.net:443") conn.request("POST", "/1/messages.json", urllib.urlencode({ "token": PUSH_TOKEN, "user": PUSH_USER, "message": msg, }), { "Content-type": "application/x-www-form-urlencoded" }) conn.getresponse() return while True: light.off() old_time = time() pir.wait_for_motion() new_time = time() if new_time - old_time > 1: print(PUSH_MSG) sendPush(PUSH_MSG) light.on() sleep(10) old_time = new_time pir.wait_for_no_motion() light.off() new_time = time() print("Room is empty.") |
|  | Run the program sudo python testt.py |

# Section 13
Code for Web Application

In this section, we will be setting up the code for the web applcation.

After completing section 3 – 12, you should a fully working database locally called assignmentDB that contains two tables – login and UID.

The UID table should have at least two entries of tag IDs, one admin and one not. The login table should also have at least one entry.

You should also have set up the DynamoDB tables RBM_rfidRecords and RBM_temp and the necessary components needed for the project and it should look something like this:

- Room One fritzing diagram

- Room Two fritzing diagram

- server.py

|  | Change the directory to RSC cd ~/RSC |
| --- | --- |
|  | Create a python script server.py with the code below sudo nano ~/RSC/server.py |
|  | Take note of the indentations! Replace my_hp with your own number. # Complusory line, to get flask files import RPi.GPIO as GPIO import signal from time import sleep import mysql.connector from flask import Flask, render_template,request,jsonify, session import json import mysql.connector import dynamoDB import jsonconverter as jsonc from gpiozero import LED from AWSIoTPythonSDK.MQTTLib import AWSIoTMQTTClient #connect to database mydb = mysql.connector.connect( host="localhost", user="rootadmin", passwd="root", database="assignmentDB" ) mycursor = mydb.cursor() # to write to AWS dynamoDB host = ""#Insert your host rootCAPath = "" #Insert your rootCAPath certificatePath = "" #Insert your certificatePath privateKeyPath = "" #Insert your privateKeyPath my_rpi = AWSIoTMQTTClient("RBMOnebasicPubSub") my_rpi.configureEndpoint(host, 8883) my_rpi.configureCredentials(rootCAPath, privateKeyPath, certificatePath) my_rpi.configureOfflinePublishQueueing(-1) # Infinite offline Publish queueing my_rpi.configureDrainingFrequency(2) # Draining: 2 Hz my_rpi.configureConnectDisconnectTimeout(10) # 10 sec my_rpi.configureMQTTOperationTimeout(5) # 5 sec # Connect and subscribe to AWS IoT my_rpi.connect() # function and var declaration light = LED(18) # mandatory line, create flask ojbect app = Flask(__name__) #api functions @app.route("/api/login", methods = ['POST', 'GET']) def get_login(): message = "" errmsg = "" if request.method == 'POST': user = request.form["user"] password = request.form["password"] print(user + " " + password) try: query = "SELECT * FROM login" mycursor.execute(query) res = mycursor.fetchall() for x in res: if x[2] == user and x[3] == password: session['authorise'] = True message = "true" print(authorise) data = {"message" : message, "error" : errmsg} return jsonify(data) else: message = "false" errmsg = "Username or password is incorrect" except: print("Error fetching values from database") import sys print(sys.exc_info()) data = {"message" : message, "error" : errmsg} return jsonify(data) @app.route("/api/tempHum",methods=['POST','GET']) def dash_getdata(): if request.method == 'POST': try: data = {'data': jsonc.data_to_json(dynamoDB.getDashTemp())} return jsonify(data) except: import sys print(sys.exc_info()) @app.route("/api/rfid",methods=['POST','GET']) def dash_getcarddata(): if request.method == 'POST': try: data = {'data': jsonc.data_to_json(dynamoDB.getDashRfid())} return jsonify(data) except: import sys print(sys.exc_info()) @app.route("/api/tempHum1",methods=['POST','GET']) def dash_getdata1(): if request.method == 'POST': try: data = {'data': jsonc.data_to_json(dynamoDB.getR1temp())} return jsonify(data) except: import sys print(sys.exc_info()) @app.route("/api/rfid1",methods=['POST','GET']) def dash_getcarddata1(): if request.method == 'POST': try: data = {'data': jsonc.data_to_json(dynamoDB.getR1rfid())} return jsonify(data) except: import sys print(sys.exc_info()) @app.route("/api/tempHum2",methods=['POST','GET']) def dash_getdata2(): if request.method == 'POST': try: data = {'data': jsonc.data_to_json(dynamoDB.getR2temp())} return jsonify(data) except: import sys print(sys.exc_info()) @app.route("/api/rfid2",methods=['POST','GET']) def dash_getcarddata2(): if request.method == 'POST': try: data = {'data': jsonc.data_to_json(dynamoDB.getR2rfid())} return jsonify(data) except: import sys print(sys.exc_info()) @app.route("/api/Lightsoff", methods = ['POST', 'GET']) def ACLoff(): message ="" msg = {} try: command = request.form["command"] room = request.form["room"] msg["room"] = room msg["command"] = command my_rpi.publish("RBM/LightSwitch", json.dumps(msg), 1) if command == "off": message = "off" else: message = "on" except: message = "Error changing state" import sys print(sys.exc_info()[0]) print(sys.exc_info()[1]) data = {"message" : message} return jsonify(data) #template functions @app.route("/") def dashboard(): session.clear() return render_template('login.html') @app.route("/index") def index(): authorise = session.get('authorise') if authorise: return render_template('index.html') else: return render_template('login.html') @app.route("/RoomOne") def RoomOne(): authorise = session.get('authorise') if authorise: return render_template('RoomOne.html') else: return render_template('login.html') @app.route("/RoomTwo") def RoomTwo(): authorise = session.get('authorise') if authorise: return render_template('RoomTwo.html') else: return render_template('login.html') app.secret_key="12345ThisIsASecretKey" app.run(debug=True, host='0.0.0.0', port=5000) |

- dynamoDB.py

This python file will help take data from AWS dynamoDB for the web server.

|  | Change the directory to RSC cd ~/RSC |
| --- | --- |
|  | Create a python script dynamoDB.py with the code below sudo nano ~/RSC/dynamoDB.py |
|  | Take note of the indentations! Replace my_hp with your own number. import boto3 from boto3.dynamodb.conditions import Key, Attr dynamodb = boto3.resource('dynamodb', region_name='us-west-2', aws_access_key_id='', # enter your own AWS access key aws_secret_access_key= '') # enter your own AWS secret key def getDashRfid(): try: table = dynamodb.Table('RBM_rfidRecords') response = table.scan() items = response['Items'] n=10 # limit to last 10 items data = items[:n] data_reversed = data[::-1] return data_reversed except: import sys print(sys.exc_info()) def getDashTemp(): try: table = dynamodb.Table('RBM_temp') response = table.scan() items = response['Items'] n=10 # limit to last 10 items data = items[:n] data_reversed = data[::-1] return data_reversed except: import sys print(sys.exc_info()) def getR1rfid(): try: table = dynamodb.Table('RBM_rfidRecords') response = table.query( KeyConditionExpression=Key('deviceid').eq('RBM_RoomOne'), ScanIndexForward=False ) items = response['Items'] n=10 # limit to last 10 items data = items[:n] data_reversed = data[::-1] return data_reversed except: import sys print(sys.exc_info()) def getR1temp(): try: table = dynamodb.Table('RBM_temp') response = table.query( KeyConditionExpression=Key('deviceid').eq('RBM_RoomOne'), ScanIndexForward=False ) items = response['Items'] n=10 # limit to last 10 items data = items[:n] data_reversed = data[::-1] return data_reversed except: import sys print(sys.exc_info()) def getR2rfid(): try: table = dynamodb.Table('RBM_rfidRecords') response = table.query( KeyConditionExpression=Key('deviceid').eq('RBM_RoomTwo'), ScanIndexForward=False ) items = response['Items'] n=10 # limit to last 10 items data = items[:n] data_reversed = data[::-1] return data_reversed except: import sys print(sys.exc_info()) def getR2temp(): try: table = dynamodb.Table('RBM_temp') response = table.query( KeyConditionExpression=Key('deviceid').eq('RBM_RoomTwo'), ScanIndexForward=False ) items = response['Items'] n=10 # limit to last 10 items data = items[:n] data_reversed = data[::-1] return data_reversed except: import sys print(sys.exc_info()) if __name__ == "__main__": query_data_from_dynamodb() |

- jsonconverter.py

This python file will convert the data from dynamoDB to JSON format so that it is readable by the web server.

|  | Change the directory to RSC cd ~/RSC |
| --- | --- |
|  | Create a python script jsonconverter.py with the code below sudo nano ~/RSC/jsonconverter.py |
|  | Take note of the indentations! Replace my_hp with your own number. from decimal import Decimal import json import datetime import numpy class GenericEncoder(json.JSONEncoder): def default(self, obj): if isinstance(obj, numpy.generic): return numpy.asscalar(obj) elif isinstance(obj, Decimal): return str(obj) elif isinstance(obj, datetime.datetime): return obj.strftime('%Y-%m-%d %H:%M:%S') elif isinstance(obj, Decimal): return float(obj) else: return json.JSONEncoder.default(self, obj) def data_to_json(data): json_data = json.dumps(data,cls=GenericEncoder) print(json_data) return json_data |

- Templates

If you do not already have the templates folder given:

In order for the server to run and display a webpage, we need a html page.

|  | Create a folder called ‘templates’. This file will contain all the html, css file and resources used for the webpage. |
| --- | --- |
|  | In the templates folder, create a html file called ‘index’. This html file will be the main page of the web application. |
|  | Go to https://startbootstrap.com/template-overviews/sb-admin/ and download the bootstrap. This will serve as the foundation for the layout of the web application. |
|  | In the templates folder, create another folder called ‘static’. In the static folder, create an ‘assets’ folder. This folder will contain all the resources from the bootstrap and for images. |
|  | After downloading the bootstrap, copy the ‘css’, ‘js’, ‘scss’ and vendor folders from the bootstrap file to assets. It should look like this. |
|  | In the assets folder, create an ‘img’ folder. This folder will store all the images used in the web page. |
|  | Save these two images as png to the img folder with their respective names: led_red_off; led_red_on. |
|  | Copy the following codes into the respective files. |

### Index.html

<!DOCTYPE html>

<html lang="en">

<head>

<meta charset="utf-8">

<meta http-equiv="X-UA-Compatible" content="IE=edge">

<meta name="viewport" content="width=device-width, initial-scale=1, shrink-to-fit=no">

<meta name="description" content="">

<meta name="author" content="">

<title>IOT - Dashboard</title>

<!-- Bootstrap core CSS-->

<link href="static/assets/vendor/bootstrap/css/bootstrap.min.css" rel="stylesheet">

<!-- Custom fonts for this template-->

<link href="static/assets/vendor/fontawesome-free/css/all.min.css" rel="stylesheet" type="text/css">

<!-- Page level plugin CSS-->

<link href="static/assets/vendor/datatables/dataTables.bootstrap4.css" rel="stylesheet">

<!-- Custom styles for this template-->

<link href="static/assets/css/sb-admin.css" rel="stylesheet">

</head>

<body id="page-top">

<nav class="navbar navbar-expand navbar-dark bg-dark static-top">

<a class="navbar-brand mr-1" href="#">Random Security Company 101</a>

<button class="btn btn-link btn-sm text-white order-1 order-sm-0" id="sidebarToggle" href="#">

<i class="fas fa-bars"></i>

</button>

<ul class="navbar-nav ml-auto ml-md-0" style="position:absolute; right:0">

<li class="nav-item dropdown no-arrow">

<a class="nav-link dropdown-toggle" href="#" id="userDropdown" role="button" data-toggle="dropdown"

aria-haspopup="true" aria-expanded="false">

<i class="fas fa-user-circle fa-fw"></i>

</a>

<div class="dropdown-menu dropdown-menu-right" aria-labelledby="userDropdown">

<a class="dropdown-item" href="#" data-toggle="modal" data-target="#logoutModal">Logout</a>

</div>

</li>

</ul>

</nav>

<div id="wrapper">

<!-- Sidebar -->

<ul class="sidebar navbar-nav">

<li class="nav-item active">

<a class="nav-link" href="index">

<i class="fas fa-fw fa-tachometer-alt"></i>

<span>Dashboard</span>

</a>

</li>

<li class="nav-item">

<a class="nav-link" href="RoomOne">

<i class="fas fa-fw fa-home"></i>

<span>Room One</span>

</a>

</li>

<li class="nav-item">

<a class="nav-link" href="RoomTwo">

<i class="fas fa-fw fa-home"></i>

<span>Room Two</span>

</a>

</li>

</ul>

<div id="content-wrapper">

<!-- Data Table -->

<div class="card mb-3">

<div class="card-header">

<i class="fas fa-table"></i>

Entrance Report</div>

<div class="card-body">

<div class="table-responsive">

<table class="table table-bordered" id="dataTable2" width="100%" cellspacing="0">

<thead>

<tr>

<th>Time</th>

<th>Room</th>

<th>Card Number</th>

</tr>

</thead>

<tbody id="data2">

</tbody>

</table>

</div>

</div>

<div class="card-footer small text-muted" id="updates"></div>

</div>

<!-- Chart -->

<div class="card mb-3">

<div class="card-header">

<i class="fas fa-chart-area"></i>

Temperature and Humidity Chart</div>

<div class="card-body">

<div id="myChart"></div>

</div>

<div class="card-footer small text-muted" id="updates1"></div>

</div>

<!-- Data Table -->

<div class="card mb-3">

<div class="card-header">

<i class="fas fa-table"></i>

Historical Data Table</div>

<div class="card-body">

<div class="table-responsive">

<table class="table table-bordered" id="dataTable" width="100%" cellspacing="0">

<thead>

<tr>

<th>Time</th>

<th>Room</th>

<th>Temperature</th>

<th>Humidity</th>

</tr>

</thead>

<tbody id="data">

</tbody>

</table>

</div>

</div>

</div>

<div class="card-footer small text-muted" id="updates2"></div>

</div>

<!-- /.container-fluid -->

<!-- Sticky Footer -->

<footer class="sticky-footer">

<div class="container my-auto">

<div class="copyright text-center my-auto">

<span>IOT Project 2018/19 Sem 2</span>

</div>

</div>

</footer>

</div>

<!-- /.content-wrapper -->

</div>

<!-- /#wrapper -->

<!-- Logout Modal-->

<div class="modal fade" id="logoutModal" tabindex="-1" role="dialog" aria-labelledby="exampleModalLabel" aria-hidden="true">

<div class="modal-dialog" role="document">

<div class="modal-content">

<div class="modal-header">

<h5 class="modal-title" id="exampleModalLabel">Ready to Leave?</h5>

<button class="close" type="button" data-dismiss="modal" aria-label="Close">

<span aria-hidden="true">×</span>

</button>

</div>

<div class="modal-body">Select "Logout" below if you are ready to end your current session.</div>

<div class="modal-footer">

<button class="btn btn-secondary" type="button" data-dismiss="modal">Cancel</button>

<a class="btn btn-primary" href="/">Logout</a>

</div>

</div>

</div>

</div>

<!-- Bootstrap core JavaScript-->

<script src="static/assets/vendor/jquery/jquery.min.js"></script>

<script src="static/assets/vendor/bootstrap/js/bootstrap.bundle.min.js"></script>

<!-- Core plugin JavaScript-->

<script src="static/assets/vendor/jquery-easing/jquery.easing.min.js"></script>

<!-- Custom scripts for all pages-->

<script src="static/assets/js/sb-admin.min.js"></script>

</body>

<!-- Script for google charts -->

<script type="text/javascript" src="https://code.jquery.com/jquery-3.2.1.js"></script>

<script type="text/javascript" src="https://www.gstatic.com/charts/loader.js"></script>

<script type="text/javascript">

// Load the Visualization API and the corechart package.

google.charts.load('current', { 'packages': ['corechart'] });

// Set a callback to run when the Google Visualization API is loaded.

google.charts.setOnLoadCallback(init);

function init() {

setIntervalAndExecute(function () {

getData(function (myRtnA, data) {

if (myRtnA == 'success') {

create_dataTable(data.data);

drawChart(data.data);

};

}),

getCardData(function (myRtnA, data) {

create_Table(data.data);

}),

setupdate()

}, 3000);

}

function setupdate() {

now = new Date();

document.getElementById("updates").innerHTML = now;

document.getElementById("updates1").innerHTML = now;

document.getElementById("updates2").innerHTML = now;

}

function getData(callBack) {

jQuery.ajax({

url: "/api/tempHum",

type: 'POST',

beforeSend: function () {

},

success: function (data, textStatus, xhr) {

if (data) {

var myRtnA = 'success';

} else {

var myRtnA = 'fail';

}

return callBack(myRtnA, data);

},

error: function (xhr, textStatus, errorThrown) {

console.log("opps: " + textStatus + " : " + errorThrown);

var myRtnA = "Error";

return callBack(myRtnA); // return callBack() with myRtna

}

});

}

function getCardData(callBack) {

jQuery.ajax({

url: "/api/rfid",

type: 'POST',

beforeSend: function () {

},

success: function (data, textStatus, xhr) {

if (data) {

var myRtnA = 'success';

} else {

var myRtnA = 'fail';

}

return callBack(myRtnA, data);

},

error: function (xhr, textStatus, errorThrown) {

console.log("opps: " + textStatus + " : " + errorThrown);

var myRtnA = "Error";

return callBack(myRtnA); // return callBack() with myRtna

}

});

}

function create_Table(data) {

var dataTable_html = '';

var newdata = JSON.parse(data);

for (index = 0; index < newdata.length; index++) {

datetime = (newdata[index].datetime).substring(0, 19);

jsdatetime = new Date(Date.parse(datetime));

device = newdata[index].deviceid;

room = "Room 2";

if (device == "RBM_RoomOne") {

room = "Room 1";

}

rfid = newdata[index].RFID;

var thisRow_HTML = '<tr>';

thisRow_HTML += '<td>';

thisRow_HTML += jsdatetime;

thisRow_HTML += '</td>';

thisRow_HTML += '<td>';

thisRow_HTML += room;

thisRow_HTML += '</td>';

thisRow_HTML += '<td>';

thisRow_HTML += rfid;

thisRow_HTML += '</td>';

thisRow_HTML += '</tr>';

dataTable_html += thisRow_HTML;

};

document.getElementById("data2").innerHTML = dataTable_html;

}

function drawChart(data) {

var dataTb = new google.visualization.DataTable();

dataTb.addColumn('string', 'Time');

dataTb.addColumn('number', 'Temperature');

dataTb.addColumn('number', 'Humidity');

var newdata = JSON.parse(data);

for (index = 0; index < newdata.length; index++) {

datetime = (newdata[index].datetimeid).substring(0, 19);

jsdatetime = new Date(Date.parse(datetime));

jstime = jsdatetime.toLocaleTimeString();

temp = parseInt(newdata[index].temp);

hum = parseInt(newdata[index].hum);

dataTb.addRows([[jstime, temp, hum]])

}

var chart = new google.visualization.LineChart(

document.getElementById('myChart'));

chart.draw(dataTb, {

legend: { position: 'top' }, vAxis: { baseline: 0 },

colors: ['#ffb0b0', '#98f6b3']

});

}

function create_dataTable(data) {

var dataTable_html = '';

var newdata = JSON.parse(data);

for (index = 0; index < newdata.length; index++) {

datetime = (newdata[index].datetimeid).substring(0, 19);

jsdatetime = new Date(Date.parse(datetime));

jstime = jsdatetime.toLocaleTimeString();

device = newdata[index].deviceid;

room = "Room 2";

if (device == "RBM_RoomOne") {

room = "Room 1";

}

temp = parseInt(newdata[index].temp);

hum = parseInt(newdata[index].hum);

var thisRow_HTML = '<tr>';

thisRow_HTML += '<td>';

thisRow_HTML += jstime;

thisRow_HTML += '</td>';

thisRow_HTML += '<td>';

thisRow_HTML += room;

thisRow_HTML += '</td>';

thisRow_HTML += '<td>';

thisRow_HTML += temp;

thisRow_HTML += '</td>';

thisRow_HTML += '<td>';

thisRow_HTML += hum;

thisRow_HTML += '</td>';

thisRow_HTML += '</tr>';

dataTable_html += thisRow_HTML;

};

document.getElementById("data").innerHTML = dataTable_html;

}

function setIntervalAndExecute(fn, t) {

fn();

return (setInterval(fn, t));

}

</script>

</html>

### login.html

<!DOCTYPE html>

<html lang="en">

<head>

<meta charset="utf-8">

<meta http-equiv="X-UA-Compatible" content="IE=edge">

<meta name="viewport" content="width=device-width, initial-scale=1, shrink-to-fit=no">

<meta name="description" content="">

<meta name="author" content="">

<title>IOT - Login</title>

<!-- Bootstrap core CSS-->

<link href="static/assets/vendor/bootstrap/css/bootstrap.min.css" rel="stylesheet">

<!-- Custom fonts for this template-->

<link href="static/assets/vendor/fontawesome-free/css/all.min.css" rel="stylesheet" type="text/css">

<!-- Custom styles for this template-->

<link href="static/assets/css/sb-admin.css" rel="stylesheet">

</head>

<body class="bg-dark">

<div class="container">

<div class="card card-login mx-auto mt-5">

<div class="card-header">Login</div>

<div class="card-body">

<div id="message"></div>

<form>

<div class="form-group">

<div class="form-label-group">

<input type="text" id="inputUser" class="form-control" placeholder="Username" required="required" autofocus="autofocus">

<label for="inputUser">Username</label>

</div>

</div>

<div class="form-group">

<div class="form-label-group">

<input type="password" id="inputPass" class="form-control" placeholder="Password" required="required">

<label for="inputPass">Password</label>

</div>

</div>

<a class="btn btn-primary btn-block" onclick="login()">Login</a>

</form>

</div>

</div>

</div>

<!-- Bootstrap core JavaScript-->

<script src="static/assets/vendor/jquery/jquery.min.js"></script>

<script src="static/assets/vendor/bootstrap/js/bootstrap.bundle.min.js"></script>

<!-- Core plugin JavaScript-->

<script src="static/assets/vendor/jquery-easing/jquery.easing.min.js"></script>

</body>

<script>

function login(){

var username = document.getElementById('inputUser').value;

var password = document.getElementById('inputPass').value;

checklogin(username, password);

}

function checklogin(user, pass){

var togglemessage = document.getElementById('message');

jQuery.ajax({

url: "/api/login",

type: 'POST',

data: {user:user, password:pass},

success: function(data,textStatus, jqXHR){

console.log(data.message);

if (data.message === 'true'){

window.location.replace('/index');

} else if (data.message === 'false'){

console.log(data.error);

togglemessage.innerHTML = "<h5>" + data.error + "</h5>";

}

}

})

}

</script>

</html>

### RoomOne.html

<!DOCTYPE html>

<html lang="en">

<head>

<meta charset="utf-8">

<meta http-equiv="X-UA-Compatible" content="IE=edge">

<meta name="viewport" content="width=device-width, initial-scale=1, shrink-to-fit=no">

<meta name="description" content="">

<meta name="author" content="">

<title>IOT - Dashboard</title>

<!-- Bootstrap core CSS-->

<link href="static/assets/vendor/bootstrap/css/bootstrap.min.css" rel="stylesheet">

<!-- Custom fonts for this template-->

<link href="static/assets/vendor/fontawesome-free/css/all.min.css" rel="stylesheet" type="text/css">

<!-- Page level plugin CSS-->

<link href="static/assets/vendor/datatables/dataTables.bootstrap4.css" rel="stylesheet">

<!-- Custom styles for this template-->

<link href="static/assets/css/sb-admin.css" rel="stylesheet">

</head>

<body id="page-top">

<nav class="navbar navbar-expand navbar-dark bg-dark static-top">

<a class="navbar-brand mr-1" href="index">Random Security Company 101</a>

<button class="btn btn-link btn-sm text-white order-1 order-sm-0" id="sidebarToggle" href="#">

<i class="fas fa-bars"></i>

</button>

<ul class="navbar-nav ml-auto ml-md-0" style="position:absolute; right:0">

<li class="nav-item dropdown no-arrow">

<a class="nav-link dropdown-toggle" href="#" id="userDropdown" role="button" data-toggle="dropdown"

aria-haspopup="true" aria-expanded="false">

<i class="fas fa-user-circle fa-fw"></i>

</a>

<div class="dropdown-menu dropdown-menu-right" aria-labelledby="userDropdown">

<a class="dropdown-item" href="#" data-toggle="modal" data-target="#logoutModal">Logout</a>

</div>

</li>

</ul>

</nav>

<div id="wrapper">

<!-- Sidebar -->

<ul class="sidebar navbar-nav">

<li class="nav-item">

<a class="nav-link" href="index">

<i class="fas fa-fw fa-tachometer-alt"></i>

<span>Dashboard</span>

</a>

</li>

<li class="nav-item active">

<a class="nav-link" href="RoomOne">

<i class="fas fa-fw fa-home"></i>

<span>Room One</span>

</a>

</li>

<li class="nav-item">

<a class="nav-link" href="RoomTwo">

<i class="fas fa-fw fa-home"></i>

<span>Room Two</span>

</a>

</li>

</ul>

<div id="content-wrapper">

<!-- Light Switch -->

<div class="card mb-3" style="float:left">

<div class="card-header">

<i class="fas fa-poop"></i>

Light Switch</div>

<div class="card-body">

<div id="Lights" width="100%">

<div>

<button onclick="LightToggle()">Toggle Light</button>

<img id="LED" src="static/assets/img/led_red_off.png" alt="Red LED light off">

<p>Status <a id="onoff">Off</a></br>

</p>

</div>

</div>

</div>

</div>

<!-- Data Table -->

<div class="card mb-3">

<div class="card-header">

<i class="fas fa-table"></i>

Entrance Report</div>

<div class="card-body">

<div class="table-responsive">

<table class="table table-bordered" id="dataTable2" width="100%" cellspacing="0">

<thead>

<tr>

<th>Time</th>

<th>Card Number</th>

</tr>

</thead>

<tbody id="data2">

</tbody>

</table>

</div>

</div>

<div class="card-footer small text-muted" id="updates"></div>

</div>

<!-- Chart -->

<div class="card mb-3">

<div class="card-header">

<i class="fas fa-chart-area"></i>

Temperature and Humidity Chart</div>

<div class="card-body">

<div id="myChart"></div>

</div>

<div class="card-footer small text-muted" id="updates1"></div>

</div>

<!-- Data Table -->

<div class="card mb-3">

<div class="card-header">

<i class="fas fa-table"></i>

Historical Data Table</div>

<div class="card-body">

<div class="table-responsive">

<table class="table table-bordered" id="dataTable" width="100%" cellspacing="0">

<thead>

<tr>

<th>Time</th>

<th>Temperature</th>

<th>Humidity</th>

</tr>

</thead>

<tbody id="data">

</tbody>

</table>

</div>

</div>

<div class="card-footer small text-muted" id="updates2"></div>

</div>

</div>

<!-- /.container-fluid -->

<!-- Sticky Footer -->

<footer class="sticky-footer">

<div class="container my-auto">

<div class="copyright text-center my-auto">

<span>IOT Project 2018/19 Sem 2</span>

</div>

</div>

</footer>

</div>

<!-- /.content-wrapper -->

</div>

<!-- /#wrapper -->

<!-- Logout Modal-->

<div class="modal fade" id="logoutModal" tabindex="-1" role="dialog" aria-labelledby="exampleModalLabel" aria-hidden="true">

<div class="modal-dialog" role="document">

<div class="modal-content">

<div class="modal-header">

<h5 class="modal-title" id="exampleModalLabel">Ready to Leave?</h5>

<button class="close" type="button" data-dismiss="modal" aria-label="Close">

<span aria-hidden="true">×</span>

</button>

</div>

<div class="modal-body">Select "Logout" below if you are ready to end your current session.</div>

<div class="modal-footer">

<button class="btn btn-secondary" type="button" data-dismiss="modal">Cancel</button>

<a class="btn btn-primary" href="/">Logout</a>

</div>

</div>

</div>

</div>

<!-- Bootstrap core JavaScript-->

<script src="static/assets/vendor/jquery/jquery.min.js"></script>

<script src="static/assets/vendor/bootstrap/js/bootstrap.bundle.min.js"></script>

<!-- Core plugin JavaScript-->

<script src="static/assets/vendor/jquery-easing/jquery.easing.min.js"></script>

<!-- Custom scripts for all pages-->

<script src="static/assets/js/sb-admin.min.js"></script>

</body>

<!-- Script for google charts -->

<script type="text/javascript" src="https://code.jquery.com/jquery-3.2.1.js"></script>

<script type="text/javascript" src="https://www.gstatic.com/charts/loader.js"></script>

<script type="text/javascript">

// Load the Visualization API and the corechart package.

google.charts.load('current', { 'packages': ['corechart'] });

// Set a callback to run when the Google Visualization API is loaded.

google.charts.setOnLoadCallback(init);

function init() {

setIntervalAndExecute(function () {

getData(function (myRtnA, data) {

if (myRtnA == 'success') {

create_dataTable(data.data);

drawChart(data.data);

};

}),

getCardData(function (myRtnA, data) {

create_Table(data.data);

}),

setupdate()

}, 3000);

}

function setupdate() {

now = new Date();

document.getElementById("updates").innerHTML = now;

document.getElementById("updates1").innerHTML = now;

document.getElementById("updates2").innerHTML = now;

}

function getData(callBack) {

jQuery.ajax({

url: "/api/tempHum1",

type: 'POST',

beforeSend: function () {

},

success: function (data, textStatus, xhr) {

if (data) {

var myRtnA = 'success';

} else {

var myRtnA = 'fail';

}

return callBack(myRtnA, data);

},

error: function (xhr, textStatus, errorThrown) {

console.log("opps: " + textStatus + " : " + errorThrown);

var myRtnA = "Error";

return callBack(myRtnA); // return callBack() with myRtna

}

});

}

function getCardData(callBack) {

jQuery.ajax({

url: "/api/rfid1",

type: 'POST',

beforeSend: function () {

},

success: function (data, textStatus, xhr) {

if (data) {

var myRtnA = 'success';

} else {

var myRtnA = 'fail';

}

return callBack(myRtnA, data);

},

error: function (xhr, textStatus, errorThrown) {

console.log("opps: " + textStatus + " : " + errorThrown);

var myRtnA = "Error";

return callBack(myRtnA); // return callBack() with myRtna

}

});

}

function create_Table(data) {

var dataTable_html = '';

var newdata = JSON.parse(data);

for (index = 0; index < newdata.length; index++) {

datetime = (newdata[index].datetime).substring(0, 19);

jsdatetime = new Date(Date.parse(datetime));

rfid = newdata[index].RFID;

var thisRow_HTML = '<tr>';

thisRow_HTML += '<td>';

thisRow_HTML += jsdatetime;

thisRow_HTML += '</td>';

thisRow_HTML += '<td>';

thisRow_HTML += rfid;

thisRow_HTML += '</td>';

thisRow_HTML += '</tr>';

dataTable_html += thisRow_HTML;

};

document.getElementById("data2").innerHTML = dataTable_html;

}

function drawChart(data) {

var dataTb = new google.visualization.DataTable();

dataTb.addColumn('string', 'Time');

dataTb.addColumn('number', 'Temperature');

dataTb.addColumn('number', 'Humidity');

var newdata = JSON.parse(data);

for (index = 0; index < newdata.length; index++) {

datetime = (newdata[index].datetimeid).substring(0, 19);

jsdatetime = new Date(Date.parse(datetime));

jstime = jsdatetime.toLocaleTimeString();

temp = parseInt(newdata[index].temp);

hum = parseInt(newdata[index].hum);

dataTb.addRows([[jstime, temp, hum]])

}

var chart = new google.visualization.LineChart(

document.getElementById('myChart'));

chart.draw(dataTb, {

legend: { position: 'top' }, vAxis: { baseline: 0 },

colors: ['#ffb0b0', '#98f6b3']

});

}

function create_dataTable(data) {

var dataTable_html = '';

var newdata = JSON.parse(data);

for (index = 0; index < newdata.length; index++) {

datetime = (newdata[index].datetimeid).substring(0, 19);

jsdatetime = new Date(Date.parse(datetime));

jstime = jsdatetime.toLocaleTimeString();

temp = parseInt(newdata[index].temp);

hum = parseInt(newdata[index].hum);

var thisRow_HTML = '<tr>';

thisRow_HTML += '<td>';

thisRow_HTML += jstime;

thisRow_HTML += '</td>';

thisRow_HTML += '<td>';

thisRow_HTML += temp;

thisRow_HTML += '</td>';

thisRow_HTML += '<td>';

thisRow_HTML += hum;

thisRow_HTML += '</td>';

thisRow_HTML += '</tr>';

dataTable_html += thisRow_HTML;

};

document.getElementById("data").innerHTML = dataTable_html;

}

function setIntervalAndExecute(fn, t) {

fn();

return (setInterval(fn, t));

}

function LightToggle() {

var toggleswitch = document.getElementById('onoff');

if (toggleswitch.innerHTML === "On") {

ACL('off');

} else {

ACL('on');

}

}

function ACL(commandtext) {

jQuery.ajax({

url: "/api/Lightsoff",

type: 'POST',

data: { command: commandtext, room: 1 },

success: function (data, textStatus, jqXHR) {

console.log(data)

var toggleswitch = document.getElementById('onoff');

var LEDimg = document.getElementById('LED')

if (data.message === "off") {

toggleswitch.innerHTML = "Off"

$('#LED').attr("src", "static/assets/img/led_red_off.png");

$('#LED').attr("alt", "Red LED light off");

} else if (data.message === "on") {

toggleswitch.innerHTML = "On";

$('#LED').attr("src", "static/assets/img/led_red_on.png");

$('#LED').attr("alt", "Red LED light on");

}

}

})

} //end ACL

</script>

</html>

### RoomTwo.html

<!DOCTYPE html>

<html lang="en">

<head>

<meta charset="utf-8">

<meta http-equiv="X-UA-Compatible" content="IE=edge">

<meta name="viewport" content="width=device-width, initial-scale=1, shrink-to-fit=no">

<meta name="description" content="">

<meta name="author" content="">

<title>IOT - Dashboard</title>

<!-- Bootstrap core CSS-->

<link href="static/assets/vendor/bootstrap/css/bootstrap.min.css" rel="stylesheet">

<!-- Custom fonts for this template-->

<link href="static/assets/vendor/fontawesome-free/css/all.min.css" rel="stylesheet" type="text/css">

<!-- Page level plugin CSS-->

<link href="static/assets/vendor/datatables/dataTables.bootstrap4.css" rel="stylesheet">

<!-- Custom styles for this template-->

<link href="static/assets/css/sb-admin.css" rel="stylesheet">

</head>

<body id="page-top">

<nav class="navbar navbar-expand navbar-dark bg-dark static-top">

<a class="navbar-brand mr-1" href="index">Random Security Company 101</a>

<button class="btn btn-link btn-sm text-white order-1 order-sm-0" id="sidebarToggle" href="#">

<i class="fas fa-bars"></i>

</button>

<ul class="navbar-nav ml-auto ml-md-0" style="position:absolute; right:0">

<li class="nav-item dropdown no-arrow">

<a class="nav-link dropdown-toggle" href="#" id="userDropdown" role="button" data-toggle="dropdown"

aria-haspopup="true" aria-expanded="false">

<i class="fas fa-user-circle fa-fw"></i>

</a>

<div class="dropdown-menu dropdown-menu-right" aria-labelledby="userDropdown">

<a class="dropdown-item" href="#" data-toggle="modal" data-target="#logoutModal">Logout</a>

</div>

</li>

</ul>

</nav>

<div id="wrapper">

<!-- Sidebar -->

<ul class="sidebar navbar-nav">

<li class="nav-item">

<a class="nav-link" href="index">

<i class="fas fa-fw fa-tachometer-alt"></i>

<span>Dashboard</span>

</a>

</li>

<li class="nav-item">

<a class="nav-link" href="RoomOne">

<i class="fas fa-fw fa-home"></i>

<span>Room One</span>

</a>

</li>

<li class="nav-item active">

<a class="nav-link" href="RoomTwo">

<i class="fas fa-fw fa-home"></i>

<span>Room Two</span>

</a>

</li>

</ul>

<div id="content-wrapper">

<!-- Data Table -->

<div class="card mb-3">

<div class="card-header">

<i class="fas fa-table"></i>

Entrance Report</div>

<div class="card-body">

<div class="table-responsive">

<table class="table table-bordered" id="dataTable2" width="100%" cellspacing="0">

<thead>

<tr>

<th>Time</th>

<th>Card Number</th>

</tr>

</thead>

<tbody id="data2">

</tbody>

</table>

</div>

</div>

<div class="card-footer small text-muted" id="updates"></div>

</div>

<!-- Chart -->

<div class="card mb-3">

<div class="card-header">

<i class="fas fa-chart-area"></i>

Temperature and Humidity Chart</div>

<div class="card-body">

<div id="myChart"></div>

</div>

<div class="card-footer small text-muted" id="updates1"></div>

</div>

<!-- Data Table -->

<div class="card mb-3">

<div class="card-header">

<i class="fas fa-table"></i>

Historical Data Table</div>

<div class="card-body">

<div class="table-responsive">

<table class="table table-bordered" id="dataTable" width="100%" cellspacing="0">

<thead>

<tr>

<th>Time</th>

<th>Temperature</th>

<th>Humidity</th>

</tr>

</thead>

<tbody id="data">

</tbody>

</table>

</div>

</div>

<div class="card-footer small text-muted" id="updates2"></div>

</div>

</div>

<!-- /.container-fluid -->

<!-- Sticky Footer -->

<footer class="sticky-footer">

<div class="container my-auto">

<div class="copyright text-center my-auto">

<span>IOT Project 2018/19 Sem 2</span>

</div>

</div>

</footer>

</div>

<!-- /.content-wrapper -->

</div>

<!-- /#wrapper -->

<!-- Logout Modal-->

<div class="modal fade" id="logoutModal" tabindex="-1" role="dialog" aria-labelledby="exampleModalLabel" aria-hidden="true">

<div class="modal-dialog" role="document">

<div class="modal-content">

<div class="modal-header">

<h5 class="modal-title" id="exampleModalLabel">Ready to Leave?</h5>

<button class="close" type="button" data-dismiss="modal" aria-label="Close">

<span aria-hidden="true">×</span>

</button>

</div>

<div class="modal-body">Select "Logout" below if you are ready to end your current session.</div>

<div class="modal-footer">

<button class="btn btn-secondary" type="button" data-dismiss="modal">Cancel</button>

<a class="btn btn-primary" href="/">Logout</a>

</div>

</div>

</div>

</div>

<!-- Bootstrap core JavaScript-->

<script src="static/assets/vendor/jquery/jquery.min.js"></script>

<script src="static/assets/vendor/bootstrap/js/bootstrap.bundle.min.js"></script>

<!-- Core plugin JavaScript-->

<script src="static/assets/vendor/jquery-easing/jquery.easing.min.js"></script>

<!-- Custom scripts for all pages-->

<script src="static/assets/js/sb-admin.min.js"></script>

</body>

<!-- Script for google charts -->

<script type="text/javascript" src="https://code.jquery.com/jquery-3.2.1.js"></script>

<script type="text/javascript" src="https://www.gstatic.com/charts/loader.js"></script>

<script type="text/javascript">

// Load the Visualization API and the corechart package.

google.charts.load('current', { 'packages': ['corechart'] });

// Set a callback to run when the Google Visualization API is loaded.

google.charts.setOnLoadCallback(init);

function init() {

setIntervalAndExecute(function () {

getData(function (myRtnA, data) {

if (myRtnA == 'success') {

create_dataTable(data.data);

drawChart(data.data);

};

}),

getCardData(function (myRtnA, data) {

create_Table(data.data);

}),

setupdate()

}, 3000);

}

function setupdate() {

now = new Date();

document.getElementById("updates").innerHTML = now;

document.getElementById("updates1").innerHTML = now;

document.getElementById("updates2").innerHTML = now;

}

function getData(callBack) {

jQuery.ajax({

url: "/api/tempHum2",

type: 'POST',

beforeSend: function () {

},

success: function (data, textStatus, xhr) {

if (data) {

var myRtnA = 'success';

} else {

var myRtnA = 'fail';

}

return callBack(myRtnA, data);

},

error: function (xhr, textStatus, errorThrown) {

console.log("opps: " + textStatus + " : " + errorThrown);

var myRtnA = "Error";

return callBack(myRtnA); // return callBack() with myRtna

}

});

}

function getCardData(callBack) {

jQuery.ajax({

url: "/api/rfid2",

type: 'POST',

beforeSend: function () {

},

success: function (data, textStatus, xhr) {

if (data) {

var myRtnA = 'success';

} else {

var myRtnA = 'fail';

}

return callBack(myRtnA, data);

},

error: function (xhr, textStatus, errorThrown) {

console.log("opps: " + textStatus + " : " + errorThrown);

var myRtnA = "Error";

return callBack(myRtnA); // return callBack() with myRtna

}

});

}

function create_Table(data) {

var dataTable_html = '';

var newdata = JSON.parse(data);

for (index = 0; index < newdata.length; index++) {

datetime = (newdata[index].datetime).substring(0, 19);

jsdatetime = new Date(Date.parse(datetime));

rfid = newdata[index].RFID;

var thisRow_HTML = '<tr>';

thisRow_HTML += '<td>';

thisRow_HTML += jsdatetime;

thisRow_HTML += '</td>';

thisRow_HTML += '<td>';

thisRow_HTML += rfid;

thisRow_HTML += '</td>';

thisRow_HTML += '</tr>';

dataTable_html += thisRow_HTML;

};

document.getElementById("data2").innerHTML = dataTable_html;

}

function drawChart(data) {

var dataTb = new google.visualization.DataTable();

dataTb.addColumn('string', 'Time');

dataTb.addColumn('number', 'Temperature');

dataTb.addColumn('number', 'Humidity');

var newdata = JSON.parse(data);

for (index = 0; index < newdata.length; index++) {

datetime = (newdata[index].datetimeid).substring(0, 19);

jsdatetime = new Date(Date.parse(datetime));

jstime = jsdatetime.toLocaleTimeString();

temp = parseInt(newdata[index].temp);

hum = parseInt(newdata[index].hum);

dataTb.addRows([[jstime, temp, hum]])

}

var chart = new google.visualization.LineChart(

document.getElementById('myChart'));

chart.draw(dataTb, {

legend: { position: 'top' }, vAxis: { baseline: 0 },

colors: ['#ffb0b0', '#98f6b3']

});

}

function create_dataTable(data) {

var dataTable_html = '';

var newdata = JSON.parse(data);

for (index = 0; index < newdata.length; index++) {

datetime = (newdata[index].datetimeid).substring(0, 19);

jsdatetime = new Date(Date.parse(datetime));

jstime = jsdatetime.toLocaleTimeString();

temp = parseInt(newdata[index].temp);

hum = parseInt(newdata[index].hum);

var thisRow_HTML = '<tr>';

thisRow_HTML += '<td>';

thisRow_HTML += jstime;

thisRow_HTML += '</td>';

thisRow_HTML += '<td>';

thisRow_HTML += temp;

thisRow_HTML += '</td>';

thisRow_HTML += '<td>';

thisRow_HTML += hum;

thisRow_HTML += '</td>';

thisRow_HTML += '</tr>';

dataTable_html += thisRow_HTML;

};

document.getElementById("data").innerHTML = dataTable_html;

}

function setIntervalAndExecute(fn, t) {

fn();

return (setInterval(fn, t));

}

</script>

</html>

- Run the program

|  | In your Raspberry Pi, open the Terminal. |
| --- | --- |
|  | Change the directory to RSC cd ~/RSC |
|  | Run dht.py sudo python dht.py Leave the script running. |
|  | Open another Terminal |
|  | Run read_display.py sudo python read_display.py Leave the script running. |
|  | Open another Terminal |
|  | If you are using the Raspberry Pi that is hosting Room One, run lights.py sudo python lights.py If you are using the Raspberry Pi that is hosting Room Two, run testt.py sudo python testt.py Leave the script running. |
|  | Open another Terminal |
|  | Run server.py sudo python server.py |
|  | Go to 0.0.0.0:5000 and see the web application. Replace 0.0.0.0 with your Raspberry Pi’s IP address. |

-- End of CA2 Step-by-step tutorial --