Ping Pong Score Board

Circuit Layout

- SPST Switch for Changing Game Length

  - GND

  - Pin A8

- SPST Switch for Serve Length

  - GND

  - Pin A9

- SPST Switch for Personal Names vs. Player 1 & 2

  - GND

  - Pin A10

- Score Point Buttons (All Momentary NO pushbuttons)

  - GND (all of them)

  - Pin 2 – Reset Match

  - Pin 3 – Switch Serve back and forth

  - Pin 4 – Scored Point Player 1

  - Pin 5 – Remove Point Player 1

  - Pin 6 – Scored Point Player 2

  - Pin 7 – Remove Point Player 2

- Connections from Arduino to FX Sound Board Pins

  - SoundStartup Pin 36   to FX 0		Startup Ping Pong ball bouncing

  - SoundChangeServe Pin 37 to FX 1           Switch Serve

  - SoundNewGame Pin 38 to FX 2                New Game, Shall We Play a Game

  - SoundGameOver Pin 39 to FX 3                 Game Over + Cheering

  - SoundWinnerIs1 Pin 40 to FX 4                  And the Winner is Player 1

  - SoundWinnerIs2 Pin 41 to FX 5                    And the Winner is Player 2

  - SoundUndo Pin 42 to FX 6                     	     Point was a mistake “Oh No”

  - SoundWinnerIsJoan Pin 43 to FX 7                And the winner is “Joan”

  - SoundWinnerIsIrwin Pin 44 to FX 8               same for “Irwin”

  - SoundLetsPlay Pin 45 to FX 9		Startup “Let’s Play Ping Pong”

  - Pin 8 to ACT on FX		Remains Low while FX is busy playing

  - GND to GND

  - 3 V from Arduino

- Connections to AdaFruit 16x32 LED Matrix (note this is for the Arduino Mega)

  - CLK to Pin 11

  - OE to Pin 9

  - LAT to Pin 10

  - A to A0

  - B to A1

  - C to A2

  - GND to Arduino GND (four connections)

  - Separate 5v 2a Power Supply Power connector

- AdaFruit 16x2 LCD with backpack  (Shows settings)

  - VIN to 5v

  - GND to GND

  - SDA and SLC pins

- AdaFruit 1.2 inch 7-Segment LED (Shows Score)

  - Vin and IO (2nd power input) to 5v Pin

  - SDA and SLC connected to Arduino SDA and SLC pin

- AdaFruit .58 inch 7-Segment LED (Shows Games Score)

  - Vin to 5v Pin

  - SDA and SLC connected to Arduino SDA and SLC pin

- Speakers (8 ohm)

  - Connected to FX Sound Board speaker contacts (L&R)

- SPST switch for On / Off controlling extension cord that power supply (9v) for Arduino and (5v) for LED matrix

- Big Pushbuttons (Arduino pins 4&6) have 5v LED’s to light up

  - Connected to extra 5V and GND from RGB 16x32 matrix power supply